Journal of Crop and Weed, 10(2):441-444(2014)

Fate and behaviour of azoxystrobin in chilli by using liquid chromatography with mass spectroscopy

# GHOSH, 1S. MUKHARJEE, 2A. BHATTACHARYYA,

  - BHATTACHARYYA AND S. ROY

Dept. of Agricultural Chemicals, B.C.K.V., Mohanpur, Nadia, West Bengal

1  Dept. of Agricultural Chemistry and Soil Science, University of Calcutta, Kolkata

2  Dept. of Chemistry, University of Kalyani, Nadia, West Bengal Received: 24-07-2014, Revised: 25-09-2014, Accepted: 30-09-2014 ABSTRACT

A liquid chromatography mass spectrometry (LC/MS-MS) method for analysis of residual azoxystrobin in chilli was developed in

present work. Azoxystrobin is generally a systemic fungicide applied in chilli to control anthracnose and powdery mildew diseases. The method was based on extraction with 1:1 ethyl acetate: 10% aq. NaCl solution, clean-up by PSA, GCB and florisil, evaporated in N2 evaporator, followed by volume make up with Acetonitrile and analysed by LC/MS-MS in multiple reaction monitoring (MRM) mode. Recoveries from fortified samples were found in the range from 90% to 92%. The limit of quantification (LOQ) of the method was 0.01 ppm. The half life value of azoxystrobin in chilli was determined as 0.73 to 0.80 days.

Keywords: Azoxystrobin, half-life, LC-MS/MS, LOQ