Journal of Crop and Weed, 10(2):472-479(2014)

Analytical method, validation and degradation kinetics of Fluroxypyr- Meptyl in onion using gas chromatography-tandem mass spectrometry

S. MUKHERJEE,1S. MUKHERJEE, 1A. KUNDU, 2G. K. DAS AND 1A. BHATTACHARYYA

Dept. of Chemistry, Visva Bharati, Santiniketan-731235, West Bengal

1Dept of Ag. Chemicals, BCKV, Mohanpur-741252, West Bengal

2Dept. of Ag. Chemistry and Soil Science, Calcutta University, Kolkata-700019 Received: 20-08-2014, Revised: 25-09-2014, Accepted: 30-09-2014 ABSTRACT

A simple, fast and inexpensive analytical method was developed and validated for screening of Fluroxypyr-Meptyl, a selective

post-emergent systemic broadleaf herbicide in onion (plant leaf, bulb) and soil using GC-MS/MS. The method comprised of extraction with ethyl acetate using matrix solid phase dispersion method followed by column chromatographic and dispersive solid phase extraction cleanup and quantified in GC-MS/MS. The method was validated in concentration ranging from 0.02-0.1

µg g-1. The recovery of Fluroxypyr-Meptyl in onion (all substrates) and soil was ranging from 88-98% at different levels of analytes with RSD (HorRat< 0.51) of 8-15%. The global uncertainty was calculated at 0.02 µg g-1. In order to evaluate in safety use in India and degradation kinetics, a field study was conducted following the validated method.

Keywords: Degradation, fluroxypyr-Meptyl, GC-MS/MS, onion, validation, uncertainty,